INTRODUCTION

As  the SYSTIME hardware is not identical to that supplied

and supported by DEC, modifications have to be made to the

standard  DEC  Operating  Systems  to  support them.  This

document  describes  those  changes  as applied to RSX-11M

V3.2  and  RSX-11M-PLUS  V1.0.    It is divided into three

sections:-

Changes for the Storage Module Drives (SMDs)

Changes for the AMPEX Cartridge Drives

Changes for the SYSTIME MKII/MKIII terminals

Each  section  describes  briefly  the  relevant  hardware

differences and then the patches that are necessary.

Hard  copies  of  all the patches are stored in the binder

marked "SYSTIME Hardware Patches for RSX-11M/M-PLUS".

CHAPTER 1

Storage Module Drives

a)      Hardware Differences.

The  Storage  Module Drives are built to look like the DEC

RP02/RP03   drives.    The  differences  (as  far  as  the

operating  system  is  concerned)  lies  in  the number of

Sectors  per  Track,  the number of Tracks per Surface and

the  number of Surfaces per Drive.  The second of these is

rarely  used  directly,  but instead the number of Sectors

(Blocks)  per  Drive  (the  product of the above three) is

usually  used.    For  DEC  and SYSTIME disks these values

are:-

Disk Type     Surfaces     Sectors      Blocks

per Drive    per Track    per Drive

DEC RP02      20.          10.          40000.

DEC RP03      20.          10.          80000.

SYS 16MB      5.           32.          32000.

SYS 32MB      5.           32.          64000.

SYS 64MB      5.           32.          128000.

SYS 128MB     19.          32.          243200.

SYS 256MB     19.          32.          486000.

To  distinguish between the RP02 and RP03 drives, there is

a  special  bit  (the 'RP03 bit') set in one of the device

registers  (the  Device Status Register).  This is used to

distinguish either between 32MB and 64MB drives or between

128MB  and  256MB drives.  Because of the way this is used

by  SAV  (see  later)  RSX  systems  should really only be

configured with:-

all 16MB drives, or

a mix of 32MB drives and 64MB drives, or

a mix of 128MB drives and 256MB drives

plus, of course, any non-SMD drives.  This restriction can

be overcome if necessary, but is not recommended.

The  following  Software  Patches  are for the most common

combination,  viz.  32MB  and  64MB  drives.   The changes

necessary  for  the  other  drives should be obvious.  The

patches   have   been   tested   for   both   RSX-11M  and

RSX-11M-PLUS.    As  all SYSTIME systems are assumed to be

running on SMDs and not on RP11s these patches are already

incorporated into the files on the distribution disk.

b)      RSX-11M

Four files need to be changed for RSX-11M as follows:-

1)      [11,10]DPDRV.MAC

This  is  the  source file for the driver for the RP11/SMD

drives  during  online  operation.   It converts a logical

block  number to a cylinder/track/block address, and it is

this calculation that must be changed:-

After 10$: after DPINI change

DIV     #100.,R1

to

DIV     #80.,R1

and

DIV     #5,R1

to

DIV     #32.,R1

2)      [200,200]SGNPER.CMD

This  is  the  file which creates the UCBs for the devices

during  System Generation.  As the DP UCB contains a block

count for the device this must be modified for SMDs:-

After .DP30: change

.SETN TEMPN4 34200

to

.SETN TEMPN4 172000

and after .DP10: change

.SETN TEMPN4 116100

to

.SETN TEMPN4 175000

3)      [12,10]SPCDRV.MAC

SAV and BOO do not use the standard system driver, but use

a  special  driver  contained  in this file, which must be

changed for SMDs as in 1:-

After $DPDRV change

MOV     #10.,R4

to

MOV     #32.,R4

and after 30$: after $DPDRV: change

MOV     #20.,R4

to

MOV     #5.,R4

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC SPCDRV=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]SPCDRV

LBR SAV/RP/-EP=SPCDRV

4)      [12,10]SAVSUB.MAC

When  a  'Saved System Image' is booted, SAV sizes all the

hardware  on the system before RSX-11M starts up.  For the

RP11/SMD  controller  it  does  this  by setting a default

number  of  blocks  and  then,  if  the 'RP03 bit' is set,

doubling  that  number.  This value is then put in the UCB

for  the  relevant  disk.   (Note that this means that for

systems  using  16MB  drives  this file must be altered or

else,  no  matter what was specified at System Generation,

the   system   will   be   brought  up  with  32MB  drives

configured.)

After DEVSIZ: change

0,40000.

to

0,175000

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC SAVSUB=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]SAVSUB

LBR SAV/RP/-EP=SAVSUB

c)      RSX-11M-PLUS

The  changes  required  for  RSX-11M-PLUS  are  similar in

effect  to  those for RSX-11M, but the relevant code is in

different modules:-

1)      [200,200]SGNMAS.CMD

The  disk  UCBs  on  RSX-11M-PLUS contain all three values

given  in  our  table  above, so only they, and not the DP

driver, need be altered.

After .DP30: change

.SETN ZN2 34200

to

.SETN ZN2 172000

and

.SETN ZN2 116100

to

.SETN ZN2 175000

and

.BYTE 10.,20.

to

.BYTE 32.,5.

2)      [12,10]SAVDP.MAC

The  special  drivers for SAV and BOO have been split into

separate modules, of which this one handles the RP11/SMD.

After 1$: after DPCSR: change

DIV     #<10.*20.>,R2

to

DIV     #<32.*5.>,R2

and

DIV     #10.,R2

to

DIV     #32.,R2

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC SAVDP=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]SAVDP

LBR SAV/RP/-EP=SAVDP

3)      [27,10]HRSIZ.MAC

SAV  now  invokes  a module to do the disk sizing (sharing

with the reconfigurer task).

In the device table change

SIZE    0,116100,10.,20.,200.

to

SIZE    0,175000,32.,5.,400.

and

SIZE    1,34200,10.,20.,400.

to

SIZE    1,172000,32.,5.,800.

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC HRSIZ=[1,1]EXEMC/ML,[11,10]RSXMC,[27,10]HRPRE,HRSIZ

LBR OLR/RP/-EP=HRSIZ

4)      [12,10]DEVOV.MAC

On  RSX-11M-PLUS  the  'DEV'  command displays the type of

disk  drive  for  RP11s.  To get this to display correctly

for SMDs:-

After TYPE: change

RP02,RP03

to

SM32,SM64

and after DPSET: change

MOV     #RP02,R1

to

MOV     #SM32,R1

and

CMP     #40000.,U.CW3(R4)

to

CMP     #175000,U.CW3(R4)

and

MOV     #RP03,R1

to

MOV     #SM64,R1

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC DEVOV=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]DEVOV

LBR SAV/RP=DEVOV

CHAPTER 2

AMPEX Cartridge Drives

a)      Hardware.

The  AMPEX  Cartridge  Drives  are  identical  to the RK11

Cartridge  Drives,  except  that  they  have twice as many

Tracks  per  Surface.    The effect of this, as far as the

software  is  concerned,  lies in the contents of the Disk

Address Register.  For the RK11 bits 5 to 12 represent the

cylinder  number, and bits 13 to 15 the drive number;  for

the  AMPEX bits 5 to 13 represent the cylinder number, and

bits 14 to 15 the drive number.

As  SYSTIME  systems  are  sold  with  both RK11 and AMPEX

drives  (though they must not be mixed on the same system)

these  patches  are  not included in the basic software on

the  distribution  disks  (though,  for  RSX-11M,  variant

'AMPEX'  sources  exist  on the disk).  These patches have

only been tested for RSX-11M.

b)      RSX-11M.

There are three changes necessary for the AMPEX mods:-

1)      [11,10]DKDRV.MAC.

As  with the SMDs this is the module that holds the driver

for online operation.  For AMPEX drives:-

After 20$: after DKINI: change

ASR     R2

to

ASRB    R0

Note  that  the  version  of this file on the Distribution

Disk  has  this  change  conditionalised  under the symbol

DK$AMX.

2)      [12,10]SPCDRV.MAC.

As  for SMDs this module contains a special driver for use

by SAV and BOO.  For AMPEX drives:-

After DKLLEN: change

BISB    DKFUN+1,$DKDAR+1

to

ASLB    DKFUN+1

BISB    DKFUN+1,$DKDAR+1

ASRB    DKFUN+1

and after DK2: change

BIC     #17777,RKDA(R5)

to

BIC     #37777,RKDA(R5)

and after DKEXIT: change

BIC     #17777,R4

to

BIC     #37777,R4

and change the first occurrence of

ROL     R4

to

;       ROL     R4

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC SPCDRV=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]SPCDRV

LBR SAV/RP/-EP=SPCDRV

Note  that  on  the  Distribution Disk a re-assembled file

exists  under  the name [1,24]SPCDRVAMX.OBJ which may just

be replaced in the library.

3)      The UCBs.

As  for SMDs the UCB for an RK11/AMPEX contains the number

of  blocks  on  the  device.  During System Generation the

user should pause to edit the 'Executive files' and change

the value in the UCB for each drive from 4800. to 9600.

A comment is output by SYSGEN.CMD on the Distribution Disk

just  before the 'Do you wish to edit any of the Executive

files' question reminding the user of what to do for AMPEX

drives.    (Note  that this comment will not appear if the

SYSGEN is being done with a Saved Answer File.)

c)      RSX-11M-PLUS

Here again the changes are much the same as for RSX-11M.

1)      [11,10]DKDRV.MAC

After 15$: after DKINI: change

ASH     #13.,R1

to

ASH     #14.,R1

2)      [12,10]SAVDK.MAC

After DKLLEN: change

BISB    DKFUN+1,$DKDAR+1

to

ASLB    DKFUN+1

BISB    DKFUN+1,$DKDAR+1

ASRB    DKFUN+1

and after DK2: change

BIC     #17777,RKDA(R5)

to

BIC     #37777,RKDA(R5)

and after DKEXIT: change

BIC     #17777,R4

to

BIC     #37777,R4

and

ASH     #-5,R4

to

ASH     #-6,R4

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC SAVDK=[1,1]EXEMC/ML,[11,10]RSXMC/PA:1,[12,10]SAVDK

LBR SAV/RP/-EP=SAVDK

3)      [27,10]HRSIZ.MAC.

As for the SMDs this sets up the size in the UCBs.

In the device table change:-

SIZE    0,4800.,12.,2,200.

to

SIZE    0,9600.,12.,2,400.

This  file  should  then  be assembled and replaced in the

relevant library, like so:-

SET /UIC=[1,24]

MAC HRSIZ=[1,1]EXEMC/ML,[11,10]RSXMC,[27,10]HRPRE,HRSIZ

LBR SAV/RP/-EP=HRSIZ

4)      The UCBs.

As for RSX-11M.

CHAPTER 3

SYSTIME MKII/MKIII Terminals

a)      Hardware.

The   full-duplex   terminal   driver   on   RSX-11M   and

RSX-11M-PLUS  supports a wide range of terminal types, for

use  in  identifying  terminals  (SET  /VT52), for setting

default  terminal characteristics (SET /VT52=TT0:) and for

'terminal-independent  cursor positioning'.  As the Cursor

Positioning  Sequence for the SYSTIME MKII is different to

other  terminals (30.,X+32.,Y+64.) extra code is needed to

support  it.    In addition a SET /MKII interface has been

added  to  MCR/TTDRV  to set the terminal to 80 characters

wide, 25 lines long and to be a CRT.

At this level SYSTIME MKIIIs are identical to MKIIs and so

the  same interface can be used for each.  As most SYSTIME

systems  are sold with SYSTIME terminals these changes are

already  in  the  software  on  the Distribution Disk (for

RSX-11M).  The changes have been tested for RSX-11M.

b)      RSX-11M and RSX-11M-PLUS.

Although  the patches have so far only been implemented on

RSX-11M, the files are almost the same on RSX-11M-PLUS and

so the changes required should be identical.  In each case

four files need modification:-

1)      [12,10]SPROVFDT.MAC (SPROV.MAC on RSX-11M-PLUS)

This module processes the SET /MKII=TTn: command.

In the table TRMTYP, before

.BYTE   0

insert

.ASCIZ  /MKII/

and at the end of the table KEYPRO

insert

.WORD   SETMK2

and at the end of the table TCODES

insert

.WORD   T.USR0

and immediately before

SETVT1: INC     INDEX

insert

SETMK2: INC     INDEX

During  System  Generation the Full-Duplex Terminal Driver

versions  of  various  MCR  modules  are  replaced  in the

relevant  library,  if  the FD driver was requested.  Thus

this file should be re-assembled as follows:-

SET /UIC=[1,24]

MAC SPROVFDT=[1,1]EXEMC/ML,[11,10]RSXMC,[12,10]SPROVFDT

MAC SDSOVFDT=[1,1]EXEMC/ML,[11,10]RSXMC,[12,10]SDSOVFDT

MAC SETOVFDT=[1,1]EXEMC/ML,[11,10]RSXMC,[12,10]SETOVFDT

PIP MCRTTFD.OBJ/NV=SPROVFDT.OBJ,SDSOVFDT.OBJ,SETOVFDT.OBJ

2)      [12,10]SDSOVFDT.MAC (SDSOV.MAC on RSX-11M-PLUS)

This module handles the MCR command SET /MKII.

After T14: add

T15:    .ASCIZ  /MKII/

At the end of the table TTYTYP insert

.WORD   T15

At the end of the table TCODES before

TCODLN=.-TCODES

insert

.WORD   T.USR0

The file should be re-assembled as above.

3)      [11,10]TTDAT.MAC

This  module defines the types of terminals in the system,

and their cursor positioning sequences.

After SPDZ: change

TTPHI   ==      T.SCR0

to

TTPHI   ==      T.USR0

and

TERM    T.VT05  WID=72.,...,SCP=1,CUP=1

to

TERM    T.VT05  WID=72.,...,SCP=1,CUP=0

and after the line

TERM    T.SCR0  WID=132.,LEN=66.

insert

TERM    T.USR0  WID=80.,LEN=25.,SCP=1,CUP=1

and change the table CP1:: to

CP1::

.BYTE   24.

.BYTE   30.,0,0

4)      [11,10]TTSUB.MAC.

This    is    the    module   that   actually   does   the

'terminal-independent cursor psoitioning'.

Change the code after CPOS1 to:-

ADD     #1001*40,R2

MOVB    R2,CP1+2

SWAB    R2

MOVB    R2,CP1+3

MOV     #3,R2

MOV     #CP1+1,R3

TST     (SP)+

CLZ

BPL     3$

DEC     R3

INC     R2

3$:     RETURN

Systime Limited

Processing errors

Page   Error description

1   Cannot decipher Printer Control block keyword, block ignored