Technical Notes on RSX-11M-PLUS                         Page 1

INTRODUCTION

We  have  recently received documentation and software for

RSX-11M-PLUS.  While no exhaustive work has been done here

on  the  system,  I have modified it to run on SYSTIME SMD

disk drives and it appears to work successfully, and to be

in  a  fit  state  for  selling to customers.  These notes

merely  record  a few points I noted when working with the

system.  In addition it should be borne in mind, before it

is sold, that:-

i)      We have currently no spare copies of the manuals.

ii)     We will need an in-house 11/70 to support it.

Technical Notes on RSX-11M-PLUS                         Page 2

CHAPTER 1

Format of Distribution

The  version of RSX-11M-PLUS we were supplied with came on

three magtapes, as follows:-

A.      2400ft 1600 BPI tape labelled 'Tape 1'

B.      600ft 1600 BPI tape labelled 'Tape 2'

C.      600ft 1600 BPI tape labelled 'RMS-11K'

Theoretically, Tape B can be booted and loads a standalone

copy  of  BRU into memory.  This is then used to bring the

baseline  system and distribution files off Tape A.  There

were two problems with this, viz.

i)      Tape B would not boot.

ii)     The  baseline  systems on Tape A would not support

SMDs.

Thus  the  initial  System Generation had to be done under

RSX-11M  (see Chapter 2).  To obtain the Distribution Disk

contents  it was necessary to mount a scratch disk on DP1:

(for  example)  and  Tape A on the magtape and execute the

following commands:-

RUN $BRU<cr>

BRU>/BACKUP_SET:COMMON<cr>

From: MT4:<cr>

To: DP1:<cr>

INITIALIZE [Y/N] > Y<cr>

Once  that  had  succeeded, the disk had to be mounted and

the  following accounts created - [11,24], [1,54], [1,34],

[125,40],  [125,24],  [125,34]  (these  are used by Sysgen

Generation).

Finally  the  software  was  patched  for SMD support (see

previous memo).

The  files  on  Tape  C were identical to the RMS-11K V1.8

distribution   for  RSX-11M  V3.2  and  so  the  tape  was

scratched.

Note  that this procedure should not be necessary again as

there is now a 32MB disk containing the whole distribution

that can be run on SYSTIME SMDs.

Technical Notes on RSX-11M-PLUS                         Page 3

CHAPTER 2

Generating an RSX-11M-PLUS System under RSX-11M

Fortunately  RSX-11M-PLUS  and  RSX-11M  are  sufficiently

similar  that  the System Generation for the former can be

performed  while  running  under  the latter with only two

small problems:-

i)      .SETO and .SETD

The  Indirect  Command  File Processor on RSX-11M-PLUS has

had  two  new  (undocumented) directives, .SETO and .SETD,

added.  These appear to set a numeric value to be octal or

decimal,  in  the  context of a later conversion to ASCII.

As  long  as  no  more  than  7  units  of  any device are

specified,  the System Generation will proceed normally if

the  files  [200,200]SGNPER.CMD,  [200,200]SGNMAS.CMD  and

[200,200]SGNTT.CMD are edited to remove all occurrences of

these directives.

ii)     Initial Boot

The Initial Boot Code (INITL.MAC) for RSX-11M-PLUS expects

the  CSR  of  the  Boot Device to be in R0 when it starts.

This  was  not  a  requirement under RSX-11M and hence the

code  in BOO on RSX-11M does not set it up.  To get around

this,  the  RSX-11M-PLUS system should be built to include

XDT  (a  good  idea  anyway) as then the initial boot will

first  break  into  XDT,  and  the CSR can be put in R0 by

hand, as follows:-

XDT>$0/ nnnnnn 176714 <cr>

XDT>G

(176714 is the default CSR for SYSTIME SMDs).

Note  that  if this is not done the initial boot will fail

with the message:-

FATAL -- BOOT DEVICE NOT FOUND IN SYSTEM TABLES:

Again,  as we now have a running RSX-11M-PLUS system, this

procedure should no longer be necessary.

Technical Notes on RSX-11M-PLUS                         Page 4

CHAPTER 3

Odd Notes

a)      RSX-11M-PLUS  development  seems  to  have  split off from

RSX-11M development shortly after the release of V3.1 with

the result that:-

i)      RSX-11M-PLUS  System  Generation  does not support

Answer Files

ii)     RSX-11M-PLUS    is    distributed   with   various

unsupported  utilities  including  TECO, SRD & RNO

(which  were  left  off RSX-11M V3.2 and have been

re-added by SYSTIME).

iii)    RSX-11M-PLUS   is  distributed  with  sources  for

F11ACP (left off RSX-11M V3.2).

There  may  be  other  hangovers  that  will  be seen as a

restriction to users familiar with RSX-11M V3.2.

Of the unsupported software the only program that looks of

much  general  use  is  SOS  (the VAX editor) which I have

transferred  to RSX-11M.  There are also a large number of

help  files  on  RSX-11M-PLUS  which can be transferred to

RSX-11M with only minor changes.

I also hope to investigate the possibility of copying over

the  F11ACP  and FCS sources and modifying them to RSX-11M

(these could be of much use to users such as Siemens).  In

case I do not manage to complete this the procedure I have

in mind is:-

Assemble the RSX-11M-PLUS module

Compare it (with VCO) to the RSX-11M module

If they are the same, then copy source over direct

Else  dis-assemble  the RSX-11M module and compare

with the RSX-11M-PLUS source

Given the closeness of RSX-11M and RSX-11M-PLUS the latter

step should not be too complicated.

I  had hoped to transfer DCL to RSX-11M, but unfortunately

(and  rather surprisingly) the sources are not distributed

and,   as   written,   it  requires  special  RSX-11M-PLUS

features.

b)      Running  on  RSX-11M-PLUS is remarkably similar to RSX-11M

(assuming   you   are   using  MCR  as  the  Command  Line

Interpreter)  but  there  is one minor difference that can

easily cause confusion.  All magtapes on RSX-11M-PLUS must

be mounted before they are accessed, whether they are ANSI

magtapes  or not.  Standard DOS (i.e. FLX) magtapes should

be mounted with the /FOR keyword specified.

c)      All   non-privileged   RSX-11M   tasks   should   run   on

RSX-11M-PLUS.   Thus if RSX-11M-PLUS is sold to a customer

site,  it  is probably worth adding all the non-privileged

'additional  RSX-11M  Software'  (to  be  documented  in a

separate memo) to the RSX-11M-PLUS Distribution Disk.