This  driver  is  designed  to  support  a DR-11W under

RSX-11M+ version 3.0 and later interfaced to an  ACROAMATICS

2110  Telemetry  data processor (TDP) system.  The design of

this driver follows the VAX/VMS XA driver in  direct  memory

access  (DMA) mode as closely as possible.  This was done to

ease compatibility of software access to the telemetry  sys-

tem, and its always nice to have a structure to follow.

The  driver  accepts  4 general functions, read logical

block, write logical block, set characteristics,  and  sense

characteristics.   In  addition,  the  read and write accept

subfunction codes reset, set function  bits,  and  set  time

out.   The  device is flagged as a record structured sequen-

tial device in the driver unit control  block  (UCB).   This

has  the  side effect of allowing use of read/write virtual,

but the operating system will  convert  this  to  read/write

logical and strip off all subfunction bits.

The  read  and  write  logical  functions  are  used to

transfer data via a dma channel between the external  device

and  the host PDP-11.  The transfers can have either a reset

or set function subfunction as required, and a new time  out

set if required.  The processing of the subfunctions is per-

formed prior to setting up the DMA channel, which would  al-

low  for  most hardware handshake protocols to be fulfilled.

The time out processing is performed just before setting the

go bit to start the transfer.  The transfer complete is sig-

nalled by an interrupt, which can come from  either  a  word

count  register overflow or any of the exception interrupts.

The transfer complete is signalled by placing the number  of

bytes  transferred  in  the  second word of the input output

status block (IOSB) and success (is.suc) in the  first  word

of  the IOSB.  A device time out will place a time out error

code (ie.tmo) in the frist word of the IOSB and the  CSR  in

the second word.

The driver allocates and releases unibus mapping regis-

ters (UMR's) for each data  transfer.   If  a  higher  speed

transfer is required, the driver could be modified to accept

device allocation and permanently  allocate  UMR's  at  that

time.  The interface to the TDP has a 4K word first in first

out (FIFO) buffer which allows extra processing time between

transfers before a data overrun will occur.

Since  is  is  possible  for  rogue devices to generate

spurious  interrupts,  or  as  in  this  device's  case,  to

generate  a  DMA done interrupt coincident with a frame done

interrupt when the requested transfer count exactly equals a

telemetry  frame.   In this case, a hardware race will occur

in which the ATTENTION interrupt and the DMA done  interrupt

will both be pending.  To assure correct handling of all in-

terrupts, all transfers are flagged with  an  interrupt  ex-

pected in the UCB and the interrupt handler clears the flag.

If an interrupt occurs when the flag is clear, the interrupt

is  dismissed.  All access to the interrupt expected flag is

done at hardware priority (PR5) to assure proper  synchroni-

zation.   All  setting  of  the  CSR  is also done at device

priority to assure stability.

DR-11W DRIVER FOR RSX-11M+ version 3.0 OR LATER       Page 2

For  non-DMA operations, the sense and set characteris-

tics functions are  available.   For  a  Set  characteristic

(io.stc) function, the user parameter (first word of the I/O

parameter block) has all but the lower three  bits  cleared,

and  these  bits  are  then  left shifted to the control bit

positions and or'ed into the CSR.  For a sense  characteris-

tics,  the CSR is merely read.  In both cases the entire CSR

is returned in the second word of the  IOSB.   No  interrupt

processing  is expected with these functions, and should one

occur, it would be dismissed as an unexpected interrupt.

The  following  code fragments shows a typical MACRO-11

interface to the QA driver.  The setting up of the  subfunc-

tion codes is as follows:

1.  IO.SFT=2       ;set function subfunction

2.  IO.RES=4       ;reset subfunction

3.  IO.STO=10      ;set time out subfunction

The access to the driver is shown as follows:

1.  Assign logical unit:

alun$s  #tdplun,#"QA,0  ;

bcc    setasn            ;ok ?

mov    $dsw,r0           ;get status return

return

2.  Sense characteristics

qiow$s  #io.sec,#tdplun,#tdpflg,,#driosb,,

bcc    15$               ;ok?

mov    $dsw,r0

return

15$:    tstb   driosb+1          ;check i/o

bpl    20$               ;if plus, no error

mov    driosb,r0         ;and io status

return

3.  Write logical block, set function:

wrset:  mov    r2,incnt          ;set character

count

qiow$s

#io.wlb!io.sft,#tdplun,#tdpflg,,#driosb,,<#inbuf,in

cnt,#wrsetup>

bcc    10$               ;ok ?

mov     $dsw,ro

return

10$:   tstb    driosb+1          ;transfer ok?

bmi     20$               ;minus, no

cmp     driosb+2,incnt    ;how many bytes

DR-11W DRIVER FOR RSX-11M+ version 3.0 OR LATER       Page 3

The  read  logical and set characteristics are are very

similar to the write logical and sense characteristic  exam-

ples.  The use of a new time out would be similar to the set

function subfunction, being or'ed into  the  function  code,

and  the  new time out (in seconds) as the fourth element of

the device dependant parameters.

Two command files (asm.cmd and bld.cmd) are used to as-

semble and build the driver.  These files assume the  source

code  is  in the named directory [qadrv].  The driver map is

placed in UIC [1,34] and the driver and symbol table in  UIC

[1,54].  The command file QA.CMD will load the driver at the

top of the GEN partition and configure online the controller

(QAA) and the unit (QA0).