DYC - A Mapped RX03 Handler for TSX-plus

V4a 23-Oct-90

Author:  C G Wilson, 71 Galatea St, Charleville, Australia 4470.

DYC provides an RX03 (double-sided RX02) handler capable of  reading  or

writing  RX01  and  RX02  discs.   It  may  be used under RT-11, but was

primarily designed to run in high memory ("mapped") under TSX-plus, thus

saving low memory space.

This handler is designed for Sigma/Dilog RX03 controllers, with optional

assembly  for  DEC,  MTI,  and DSD ones.  It may need minor modification

(notably to the extended  addressing  and  formatting  code)  for  other

manufacturers' boards.

Full 22-bit addressing is used where possible,  removing  the  need  for

intermediate data buffers under TSX-plus.

An SPFUN to allow formatting during  normal  system  operation  (without

requiring  access  to the I/O page) is included.  The handler may be run

as a  programme,  providing  the  formatting  options  with  no  further

programme being required.

In version 4, support for the SMS flavour of RX03 format has been  added

(courtesy of discs from Billy Youdelman).

DYC.TSX is designed for use as SY:DY.TSX, to replace the  DEC  or  S & H

DY.TSX on the system device.

CHAPTER 1

Assembly Options:

(See also later section  on  building  the  DYC  handler  for  different

systems and controllers.)

In version 4, the DEC option of DY$DD (for double density only) has been

removed.

1.1  Controller Types - Sigma/Dilog

No prefix file is required for building this flavour of the controller -

it is the default, setting the parameter C.DILOG to 1.

This version assumes 22-bit addressing (sets  DY$22B=1),  2-sided  discs

(sets  SIDES$=2), and uses the Sigma/Dilog extension to DEC's formatting

to allow both  soft  (writing  the  headers  only)  and  hard  (complete

rewriting of sector and track information) formatting.

22 bit addressing is  enabled  by  setting  the  <cs22bit>  bit  in  the

command.   An extra <TR> phase is added to all cycles requiring a memory

access.   In  this  cycle  the  high-order  address  is  passed  to  the

controller, as six bits right justified.  The bits <csma16> and <csma17>

are not used.

1.2  MTI Controller

The prefix file MTI.MAC  is  used  for  building  this  version  of  the

handler.  It sets the parameter C.MTI=1.

This version assumes 22-bit addressing (sets  DY$22B=1),  2-sided  discs

(sets SIDES$=2), and uses the MTI formatting - same as Sigma/Dilog.

22 bit addressing is  enabled  by  setting  the  <cs22bit>  bit  in  the

command.   An extra <TR> phase is added to all cycles requiring a memory

access.  In this cycle the high order  four  bits  of  the  address  are

passed to the controller, right justified.  Bits 16 and 17 still need to

be set in the command word as per a DEC 18-bit controller.

DYC - Mapped RX03 Handler for TSX-plus                          Page 1-2

Assembly Options:

1.3  DSD Controller

The prefix file DSD.MAC  is  used  for  building  this  version  of  the

handler.   It  sets  the  parameter  C.DSD=1,  and  allows 2-sided discs

(SIDES$=2).

This controller is an 18-bit one rather than a 22-bit one,  and  uses  a

completely  different formatting technique.  It has the problem that the

formatting on side 1 sets the HEAD byte to 1, and actually  checks  this

on  later disc I/O, whereas the Sigma/Dilog does not.  This means that a

disc formatted on a DSD system can be read and written on either, but  a

disc  formatted  on  a Sigma/Dilog cannot have its second side read on a

DSD system.

1.4  DEC Controller

The prefix files DEC.MAC and DEC2.MAC are  used  for  building  the  DEC

versions  of the handler.  They both set the parameter C.DEC=1.  DEC.MAC

allows  only  single-sided  discs  (SIDES$=1),   and   DEC2.MAC   allows

double-sided discs (SIDES$=2).

This controller is an 18-bit one, and is only capable of soft formatting

of discs.

1.5  16-bit Controller

The prefix file TINY.MAC is  used  for  building  this  version  of  the

handler.   It  sets the parameter C.TINY=1, and allows only single-sided

discs (SIDES$=1).

This is a special version of DYC providing  a  TSX-non-mappable  handler

for  a  16-bit controller.  It uses a buffer in its own low memory space

for transfers, copying the data  to  and  from  there  into  the  user's

address space.  It is slow, and has very limited application!

CHAPTER 2

Formatting

An SPFUN (code 200) is provided to allow formatting, with the  following

parameters:

In Buffer:

First byte:

If zero, format side zero

If non-zero, format size one

Second byte:

If zero, use single density

If non-zero, use double density

Block Number:

If zero, use soft formatting (header rewrite)

If non-zero, use hard formatting (if controller

has this ability)

Depending upon which controller is used,  the  machinations  within  the

handler are different.

Sigma/Dilog protocol (extension of DEC type):

Command:        10 + <20 if unit 1> + <400 if double dens> +  <1000  for

side  1> Followed by (in data register)  111 for soft format, or 222 for

hard format.

DSD protocol:

Command:  5 Followed by:        154 for single-density  155  for  double

density each twice into the data register.  Both sides of a disc will be

formatted if it is double sided.

DEC protocol:

Command:        10 + <20 if unit 1> + <400 if double dens> +  <1000  for

side 1> Followed by (in data register)  111 (soft format only).

CHAPTER 3

Running DYC as a Programme

If DYC is run as a programme, it provides  formatting  capabilities  for

relatively  easy use, requiring no particular privileges under TSX-plus.

[If however you have users who may not access ".TSX" files on the system

device, make a copy of DYC.TSX called DY.SAV on SY:  and run that.]

This facility is best demonstrated by showing how to get the  HELP  text

for this programme (note that there are minor differences depending upon

which assembly parameters have been used in the construction of the  DYC

handler):

.r dy.tsx

*/h

DY Extended Handler, V4, 24-Aug-90

Built for Sigma/Dilog DY controller.

Commands are:

/H (without any file) - Type this text

The device specified should be a DY floppy drive,

write-enabled.

/H - hard format;

/F - soft format;

/D - double density;

/S - both sides.

/S:0 - side 0 only (default);

/S:1 - side 1 only.

/I - initialize directory when formatted

(default 4 segments), or

/I:n for n segments (1 to 31.)

/T - tell size of disc (in blocks)

/Z - equivalent to /F/D/S/I

(/D and /S imply /F unless /H is specified.

/B - tell whether normal RX03 or Billy Youdelman's SMS format

/B:ON sets Billy's format; /B:OFF returns to normal RX03

*

[The use of /B is described in the next chapter.]

Examples:

DYC - Mapped RX03 Handler for TSX-plus                          Page 3-2

Running DYC as a Programme

.r dy.tsx

*dy:/z/h

Hard formats an RX03 floppy and initializes it.

.r dy.tsx

*dy1:/t

Tells the size of the floppy in dy1:.

.r dy.tsx

*dy1:/d/i

Soft formats an RX02 floppy in dy1: and initializes it.

CHAPTER 4

Different DY Formats

There are several formats available for 8" floppy discs, to be  readable

with the DYC handler.

RX01:  This is the single density original IBM format,  with  128  bytes

per  sector,  26 sectors per track, using tracks 1 through 76.  (Track 0

was reserved for an IBM directory, and has been  uselessly  wasted  ever

since!)  The  sector  headers  and  the  data are both written in single

density.  The number of blocks available with RX01 formatting is 494.

RX02:  This was DEC's modification  to  increase  storage  space.   This

format  still  uses  26 sectors per track, tracks 1 through 76, but each

sector, while still having its header written in single density,  writes

its  data  in double density.  This doubles the available space for data

to 988 blocks.

RX03:  When double-sided discs and drives became available, DEC did  not

move   up   to   them,  though  other  manufacturers  leaped  for  them.

Unfortunately this has led to two different formats,  as  there  was  no

standard!

Mark I:  With this format, the disc is utilized in such  a  manner  that

side  zero is exactly the same as an RX02 floppy (blocks 0 through 987),

then blocks 988 through 1971 are on the other side of the floppy in  the

same positions as the corresponding block (less 988) on side zero.  This

has the advantage that a single-sided  controller  can  still  read  the

first half of these discs.

Mark II:  [I only recently learned of the existence of this  format  via

Billy  Youdelman  in  the  USA,  who  sent me some apparently unreadable

discs!  This format has been allowed for  in  version  4  of  DYC]  This

format  alternates  heads on each track before moving to the next track:

ie starts at track 1, head 0, then goes to track 1, head 1;  then  track

2, head 0, track 2 head 1 and so forth.

It is interesting that the initial directory blocks in these two formats

are  in  exactly the same position on the disc, so you cannot tell which

format a disc was written in by examining the directory:   you  have  to

look at one of the files stored further along the disc.

Version 4 of  DYC  supports  both  of  these  formats.   When  initially

DYC - Mapped RX03 Handler for TSX-plus                          Page 4-2

Different DY Formats

assembled  and as distributed, it is set for the Mark I format, and must

be altered if the Mark II format is to be used.  This  can  be  done  in

either of the following ways:

.SET DY BILLY

.SET DY NO BILLY

These two commands set the handler to Mark II (Billy Youdelman's format)

and Mark I (the original?) respectively.

Should it be necessary to modify a handler  when  it  is  not  a  system

handler  (eg  setting  up  a bootable disc), the handler may be run as a

programme, and the /B switch used to perform the modification.  /B alone

tells  you  the  current  setting  of  the  handler;  /B:ON switches the

handler to Mark II, /B:OFF reverts to Mark I.  For example:

.RUN DY:DY.SYS

*/B

Currently SET DY NO BILLY (for normal RX03 format) is in effect.

*/B:ON

*/B

Currently SET DY BILLY (for SMS format) is in effect.

*/B:OFF

*/B

Currently SET DY NO BILLY (for normal RX03 format) is in effect.

*^C

----------------------------------------------------------------------

WARNING:  STICK TO ONE FORMAT ON YOUR OWN SYSTEM IF  POSSIBLE!   If  you

change  to read a foreign disc, CHANGE BACK AGAIN.  Due to the fact that

the directory is in the same  place  on  either  RX03  format,  you  can

UTTERLY CREAM YOUR DISCS if (by accident) you mix the formats!

----------------------------------------------------------------------

CHAPTER 5

TSMUNG - for modifying TSX plus

With the advent of TSX-plus version 6, no matter what  is  specified  in

the TSGEN file (eg DEVDEF <DY>,MAPH,HANBUF,DMA) TSX refuses to allow the

DY or the DU handler to be mapped into high memory.   This  is  fine  to

protect  people  trying  to do this with DEC or S & H handlers, but is a

nuisance when we have a legitimate handler capable of this performance.

A programme TSMUNG is provided.  This modifies the default values set up

for  the DU and/or DY devices.  It is run, using the name of the TSX.SAV

file to be modified appropriately.  To modify the DY value, append /Y to

the  filename.   (To modify the DU, append /U).  [No switches implies /U

only.]

.Run TSMUNG

*sy:tsx.sav/y

Successful TSX modification

.

or

.Run TSMUNG sy:tsx.sav/y

Successful TSX modification

.

If it should be desired to run the DU handler mapped in addition to  the

DY  handler (WARNING - again, make sure your handler is capable of being

run mapped and is not the standard DEC or S & H handler), append a /U to

the filename in addition to the /Y.  For example:

.Run TSMUNG du:tsx.sav/y/u

Successful TSX modification

.

If TSMUNG cannot find the relevant string of device  names  in  TSX,  it

will  tell  you  so.   If  this  is the case, you are probably trying to

operate upon a version of TSX different from that for which  TSMUNG  was

written (version 6.01 through 6.4).

CHAPTER 6

Building the DYC Handlers

The flavours of handler which may be built from DYC.MAC are:

(1) Sigma/Dilog (default)

(2) MTI  (prefix file MTI.MAC)

(3) DEC  (ibid, DEC.MAC)

(4) DSD  (ibid, DSD.MAC)

(5) special version for 16-bit controllers (TINY.MAC)

and for each of these the handlers may be built for any of the  flavours

of RT-11, or TSX.

Prefix files are provided as listed above for these,  with  the  proviso

that  using  no  prefix  file  produces  the  Sigma/Dilog version of the

handler.

USE OF THE WRONG HANDLER FOR YOUR PARTICULAR CONTROLLER MAY CAUSE ERRORS

OR SYSTEM CRASHES.  If you are uncertain as to which controller you have

on your system, pull it out and  check;   if  you  are  uncertain  which

flavour  of  the  handler  you  have  copied to your system, run it as a

programme and type "/H<return>".  It will  type  out  the  name  of  the

controller it was built for.

For example, to produce a handler for an MTI  controller  running  under

TSX-plus, use the following:

.macro tsxcnd+mti+dyc/obj

.link/nobit dyc/exe:dyc.tsx

The handlers built in  the  DYC  distribution  are  all  built  for  the

Sigma/Dilog  controller.   DYC.SYS  is  the  RT-11  FB/SJ  version,  for

vanilla-flavoured RT-11, DYCTIM.SYS  is  for  RT-11  FB/SJ  with  device

timeout support, and DYC.TSX is for TSX-plus.

CHAPTER 7

Notes for TSX-plus

7.1  "Filler" Buffer

Use of a mapped handler under TSX-plus places that handler up in the top

of  memory.   For  writing the "filler" sectors (sectors which would not

otherwise be written on incomplete block writes) it is necessary that  a

DY  handler  be  able  to write a zero word from itself out to the disc.

This causes a problem where the controller is only an  18-bit  one,  and

the  system  has  more  than  256k  of  memory.  To get around this, the

location 500 is used as a "dummy" buffer.  This will only cause  trouble

if  your system has a device with a vector at 500.  If this is the case,

adjust this value upwards accordingly.

7.2  MIONBF and the DEC and DSD Versions

Under TSX-plus, the DEC and DSD handlers will require MIONBF and  MIOBSZ

to be set non-zero (unless the system has only 256K of memory on it), as

the corresponding devices can only work in the first 256K of memory .

7.3  EVNBUF - Handlers Require Even Buffer Addresses

Under TSX-plus all these handlers should be set  EVNBUF  in  the  DEVDEF

section  of  TSGEN.MAC,  as otherwise it is possible to crash the system

should a buffer at an odd address be passed to the handler.  (This would

be  a  pretty unlikely happening, but is mentioned for safety.  I think,

but have not proved, that this  crashing  is  only  possible  using  the

special DY SPFUNs.)

CHAPTER 8

Distribution

DYC is usually distributed with DUCM, the internally-queueing DU handler

from the same author.

The files in the DYC distribution are:

DYC.MAC         Source

DYC.RNO         Documentation

DYC.DOC

DYC.SYS         Native RT-11 handler

DYCTIM.SYS      Handler for RT with timeout support

DYC.TSX         TSX-plus handler

DYC.COM         Assembly command file (Sigma/Dilog)

DYCTST.COM      Full test-assembly command file

MTI.MAC         Command files for non-Dilog flavours of DYC - MTI

DSD.MAC                 DSD

DEC.MAC                 DEC, 1-sided discs

DEC2.MAC                DEC, 2-sided discs

TINY.MAC                16-bit controller

TSMUNG.MAC      Programme to allow TSX DY to be mapped

TSMUNG.SAV

TIMIT.MAC       RT-11/TSX prefix files

ERLG.MAC

MMGT.MAC

TSXPRE.MAC

- o O o -

###########################################